NSTB

DATA FORMAT

(Updated November 2003)

This document describes the NSTB Data format for Test Bed Reference Station (TRS) and WAAS Reference Stations (WRS) data. The recorded data includes GPS measurements; ephemeris and almanac data; GEO measurement and WAAS broadcast data; as provided by the GPS receivers at the Reference sites.

Data files are compressed using gnu-zip. After decompression, each file contains 24 hours of data from one receiver or weather station.  The file name has the following format.

Sss_Rrr_IIII_wwww_dd

Where:

Sss	TRS/WRS site name (Acy, And, Rir …)

Rrr 	Receiver Type (For TRS only)

(Ashtech,Millenium,Gsv,Trimble,Medl,Weather,OEM4,SQM)

IIII  	Receiver ID or Weather Station ID in Hexadecimal notation

wwww	The GPS week number +1024 (1107 is week 83 for example)

NN	The day of week (00 = Sunday, 01 = Monday….06 = Saturday)

Within each file “Sss_Rrr_IIII_www_dd” data is stored as NSTB TRS-ICD Type 1, 5, 10, 20, 30, 31, or 32 messages.  The format of each of these messages is described further in this document.

The data collection process adds a header to each message collected.

The header starts with a four (4)-byte synchronization framing sequence (0xFACEDEAD). It is followed by a two byte, big endian, unsigned short integer storing the GPS week of reception, followed by a four byte, big endian unsigned integer storing the GPS milliseconds in week time of reception. The NSTB-ICD message follows.

--------------------------+ -------------+ -------------------------------+------------------------------------------+--------------------------+

<  1><  2><  3><  4> | <  5><  6>  | <  7><  8><  9>< 10>     |< 10>< 11>< 12> ... <N-1><  N>  | <  1><  2><  3><  4> |

0xFACEDEAD     | GPS Week  |  GPS RECEIVE TIME  |  NSTB-ICD Message (T1, T5 …) |      0xFACEDEAD

|                     |      (msec)                       |                                                       |

--------------------------+ -------------+ -------------------------------+------------------------------------------+--------------------------+

Data is collected and formatted on redundant UltraSparc workstations running SUN Solaris at the NSTB Master Station located at the FAA Technical Center.

All NSTB sites are posted daily on the FTP site. Only a few of the WAAS sites are posted daily on the site.If you are interested by other sites, or have any questions please contact Bill Wanner at 609-485-5514 or Nathan Vary at 609-485-5881.

The list of sites, receiver ids and surveys is available in the file: nstb.cfg

### MESSAGE TYPE 1 - SATELLITE TRACKING DATA

The receivers provide single frequency or dual frequency satellite tracking data.  This data is collected by the TRS software and formatted into a Type 1 message.  The message type 1 shall only be sent when tracking data is available.

The message includes a header, a block of bytes for each satellite tracked (dual or single frequency) and a CCITT CRC checksum.  The fields of the header provide the number of satellites tracked within dual frequency and/or single frequency channels. Also include in the header block is a “TRS Epoch Counter”, or program sequence number.  This Epoch Counter is a 16-bit unsigned integer that increments with each Message Type 1 creation and transmission; the sequence number is reinitialized after the maximum value of 65535.

Byte Order for multi-bytes fields for all NSTB messages is “Little Endian”. (Less Significant Bytes first).

Ex: TRS Epoch Counter

Value (Decimal):  4660     Value (Hex): 1234

Fields:

Byte 10 = 0x34

Byte 11 = 0x12

Flags:

Bit #   :  31--------------------------------------------------0

32 bits:   1010 1011 1100 1101 1110 1111 0001 0010

Hex    :   AB              CD            EF            12

Byte 15 : 0x12

Byte 16 : 0xEF

Byte 17: 0xCD

Byte 18: 0xAB

Sign Convention: When the Carrier Range is increasing; the Doppler is positive and the Carrier Phase is increasing.

The satellite status field is the same for a dual frequency channel, a single frequency GPS or a single frequency GEO channel.  Therefore, some of the flags may not be applicable.  In the case a bit flag is not applicable, its value should be set to one (1).  For example:

- A dual frequency channel (GPS L1/L2) : bit 15 is not applicable,

- A single frequency channel (GPS L1) : bits 1,5,6,7,9,11,12,13,15 are not applicable,

- A single frequency channel (GEO L) : bits 1,5,6,7,9,11,12,13 are not applicable,

- The distinction between a GPS and a GEO is done according to the PRN number.

The total length of the message depends on the number of tracked satellites,

Overhead (header + checksum) = 15 bytes.

Average length (8 dual frequency channels) = 407 bytes.

| BYTE | DESCRIPTION | Size Bytes | Data type |
| --- | --- | --- | --- |
| 1 | Message type = 1 | 1 | U Char |
| 2-3 | Receiver ID | 2 | U Short |
| 4-5 | Time of Validity - GPS Week | 2 | U Short |
| 6-9 | Time of Validity - milliseconds | 4 | U Long |
| 10-11 | TRS Epoch Counter | 2 | U Short |
| 12 | Number of dual frequency channels | 1 | U Char |
| 13 | Number of single frequency channels | 1 | U Char |
|  | Dual frequency channels data: 49 bytes/channel |  |  |
|  | Repeated for each dual frequency channel |  |  |
| 14 | Satellite PRN number | 1 | U Char |
| 15-18 | Satellite status flags (see bit field table next page) | 4 | U Long |
| 19-26 | L1 Pseudo-Range (PR1) (meters) | 8 | Double |
| 27-34 | L1 carrier range (meters) | 8 | Double |
| 35-42 | L2 carrier range (meters) | 8 | Double |
| 43-46 | L1/L2 differential group delay (PR2-PR1) (meters) | 4 | Float |
| 47-50 | L1 Doppler (DL1) (m/s) | 4 | Float |
| 51-54 | L2 Doppler (DL2) (m/s) | 4 | Float |
| 55-58 | L1 signal-to-noise ratio SNRL1 (dB-Hz) | 4 | Float |
| 59-62 | L2 signal-to-noise ratio SNRL2 (dB-Hz) | 4 | Float |
|  | Single frequency channels data: 29 bytes / channel |  |  |
|  | Repeated for each single frequency channel |  |  |
| n+ |  |  |  |
| 1 | Satellite PRN number | 1 | U Char |
| 2-5 | Satellite status flags ( see bit field table ) | 4 | U Long |
| 6-13 | L1 Pseudo-Range (PR1) (meters) | 8 | Double |
| 14-21 | L1 carrier range (meters) | 8 | Double |
| 22-25 | L1 Doppler (DL1)* (m/s) | 4 | Float |
| 26-29 | L1 signal-to-noise ratio SNRL1 (dB-Hz) | 4 | Float |
| 2 last | Checksum calculated on previous bytes - CCITT CRC | 2 | Short |

table 1: message type 1

The satellite status flags field is described in the following table :

| Bit | Description | Value |
| --- | --- | --- |
| 0 | L1 Tracking mode | 1 = P-code / 0 = C/A code |
| 1 | L2 Tracking mode | 1 = P-code / 0 = codeless |
| 2,3,4 | L1 Cycle slip counter | 0 to 7 |
| 5,6,7 | L2 Cycle slip counter | 0 to 7 |
| 8 | L1 Half Cycle Added | 1 = Half cycle added / 0 = no half cycle added |
| 9 | L2 Half Cycle Added | 1 = Half cycle added / 0 = no half cycle added |
| 10 | GPS L1 or GEO L Pseudo-range Reasonability Check - Shall be performed as described in ADD V.4 section 2.2.2.1.1 for GPS L1 and 2.2.2.2.1 for GEO L-Band. | 1 = passes / 0 = fails |
| 11 | GPS L2 Pseudo-range Reasonability Check - Shall be performed as described in ADD V.4 section 2.2.2.1.1 | 1 = passes / 0 = fails |
| 12 | GPS L1 and L1-L2 Differential Phase Bias Breaks Reasonability Check - Shall be performed as described in ADD V.4 section 2.2.2.1.2 | 1 = passes / 0 = fails |
| 13 | GPS differential Code measurement Reasonability Check - Shall be performed as described in ADD V.4 section 2.2.2.1.3 | 1 = passes / 0 = fails |
| 14 | GPS L1 or GEO L Pseudo-range/Carrier Phase Consistency Check - Shall be performed as described in ADD V.4 section 2.2.2.1.4 | 1 = passes / 0 = fails |
| 15 | GEO Pseudo-range dynamic behavior check - Shall be performed as described in ADD V.4 section 2.2.2.2.1 | 1 = passes / 0 = fails |
| 16 | L1 Phase Valid - As collected from receivers internal status determination. | 1 = valid / 0 = invalid |
| 17 | L2 Phase Valid - As collected from receivers internal status determination. | 1 = valid / 0 = invalid |
| 18 | L1 Pseudo-range Valid - As collected from receivers internal status determination. | 1 = valid / 0 = invalid |
| 19 | L2 Pseudo-range Valid - As collected from receivers internal status determination. | 1 = valid / 0 = invalid |
| 20-31 | Reserved for GPS receiver specific tracking status | Unused bits are null |

table 2 : satellite status flags field

### MESSAGE TYPE 5 - GEO / WAAS MESSAGE

This time critical message carries the WAAS message that has been broadcast by the GEO and has been received by a GEO-capable receiver.  The message includes a header, 1 or  more GEO satellite blocks of data and a CCITT CRC checksum.  The message type 5 shall only be sent when GEO information is available.  The header includes one field that defines the number of GEO satellites tracked.

The Time of Validity is obtained from the IFRAME message of the receiver.

Thirty-two (32) bytes are reserved to store the WAAS FEC Decoded 250 bits.  The last 6, unused bits shall be nulls.

The following table details the structure of the message and the data types for each field:

| BYTE | DESCRIPTION | Size Bytes | Data type |
| --- | --- | --- | --- |
| 1 | Message type = 5 | 1 | U Char |
| 2-3 | Receiver ID | 2 | U Short |
| 4-5 | Time of Validity - GPS Week | 2 | U Short |
| 6-9 | Time of Validity - milliseconds | 4 | U Long |
| 10 | Number of GEO Satellites tracked (max. 2) | 1 | U Char |
|  | First GEO satellite : 33 bytes |  |  |
|  | GEO PRN number | 1 | U Char |
|  | WAAS FEC Decoded bits (250 bits) | 32 | Char[32] |
|  | Second GEO satellite (if tracked) : 33 bytes |  |  |
|  | GEO PRN number | 1 | U Char |
|  | WAAS FEC Decoded bits (250 bits) | 32 | Char[32] |
| 2 last | Checksum calculated on previous bytes - CCITT CRC | 2 | Short |

table 3 : message type 5

The total length of the message depends on the number of satellites tracked.

Overhead (header + checksum) = 12 bytes

Average length (1 GEO satellite) = 45 bytes

### MESSAGE TYPE 20 - GPS EPHEMERAL DATA

The GPS receivers provide Ephemeris data to the TRS processor where this data is formatted into a message type 20.  This message contains the ephemeral data for only one tracked satellite.  Therefore, the TRS process sends one message per second until all known ephemeral data is sent.  A message type 20 is sent every 30 sec for each tracked satellite. If “bad health” occurs for a satellite, the ephemeris for that satellite is sent next.

The message includes a header, a block of ephemeral data and a CCITT CRC checksum.  The “Time of Validity” fields shall contain the same values as the “Time of Validity” of the last message type 1 sent.

The ephemeral information in message type 20 shall use the same data scaling and resolution as defined in section 2.4 of the GPS-Standard Positioning Service Signal Specification document.

Overhead (header + Checksum) = 11 bytes

Length of the message type 20 = 77 bytes

| BYTE | DESCRIPTION | Size Bytes | Data type |
| --- | --- | --- | --- |
| 1 | Message type = 20 | 1 | U Char |
| 2-3 | Receiver ID | 2 | U Short |
| 4-5 | Time of Validity - GPS Week | 2 | U Short |
| 6-9 | Time of Validity - milliseconds | 4 | U Long |
|  | Ephemeris Data for one satellite: |  |  |
| 10 | Satellite PRN number | 1 | U Char |
| 11-14 | Time navigation message was received (Absolute GPS time in seconds of week) | 4 | U Long |
| 15 | URA User range accuracy (see note 1) | 1 | U Char |
| 16 | SV Health Satellite health (discrete) | 1 | U Char |
| 17-18 | IODC Issue of Data, Clock (see note 2) | 2 | U Short |
| 19 | TGD Estimated group delay differential ( sec ) | 1 | Char |
| 20-21 | toc Clock data reference time ( sec ) | 2 | U Short |
| 22 | af2 Clock data coefficient (sec/sec2) | 1 | Char |
| 23-24 | af1 Clock data coefficient (sec/sec) | 2 | Short |
| 25-28 | af0 Clock data coefficient (sec) | 4 | Long |
| 29-32 | M0 Mean anomaly at reference time (semi-circles) | 4 | Long |
| 33-34 | n Mean motion difference from computed value (semi-circles/sec) | 2 | Short |
| 35-38 | e Eccentricity (dimensionless) | 4 | U Long |
| 39-42 | (A)1/2 Square root of semi-major axis (meters1/2) | 4 | U Long |
| 43-46 | (OMEGA)0 Longitude of ascending node of orbit plane at Weekly epoch (semi-circles) | 4 | Long |
| 47-50 | I0 Inclination angle at reference time (semi-circles) | 4 | Long |
| 51-54 |  Argument of perigee (semi-circles) | 4 | Long |
| 55-58 | OMEGADOT Rate of right ascension (semi-circles/sec) | 4 | Long |
| 59-60 | IDOT Rate of inclination angle (semi-circles/sec) | 2 | Short |
| 61-62 | Cuc Amplitude of cosine harmonic correction term to argument of latitude (rad) | 2 | Short |
| 63-64 | Cus Amplitude of sine harmonic correction term to argument of latitude (rad) | 2 | Short |
| 65-66 | Crc Amplitude of cosine harmonic correction term to orbit radius (meters) | 2 | Short |
| 67-68 | Crs Amplitude of sine harmonic correction term to orbit radius (meters) | 2 | Short |
| 69-70 | Cic Amplitude of cosine harmonic correction term to angle of inclination (rad) | 2 | Short |
| 71-72 | Cis Amplitude of sine harmonic correction term to angle of inclination (rad) | 2 | Short |
| 73-74 | toe Reference time Ephemeris (sec) | 2 | U Short |
| 75 | IODE Issue of data Ephemeris (see note 3) | 1 | U Char |
| 76-77 | Checksum CCITT CRC | 2 | Short |

parameter	# of bits	scale	range **	units

Week Number	10	1		Week

URA	4			see note 1

SV Health	6	1		discrete

TGD	8 *	2-31		sec

IODC	10			see note 2

toc	16	24	604,784	sec

af2	8 *	2-55		sec/sec2

af1	16 *	2-43		sec/sec

af0	22 *	2-31		sec

M0	32 *	2-31		semi-circles

n	16 *	2-43		semi-circles/sec

e	32	2-33	0.03	dimensionless

(A)1/2	32	2-19		meters1/2

(OMEGA)0	32 *	2-31		semi-circles

i0	32 *	2-31		semi-circles

	32 *	2-31		semi-circles

OMEGADOT	24 *	2-43		semi-circles/sec

IDOT	14 *	2-43		semi-circles/sec

Cuc	16 *	2-29		radians

Cus	16 *	2-29		radians

Crc	16 *	2-5		meters

Crs	16 *	2-5		meters

Cic	16 *	2-29		radians

Cis	16 *	2-29		radians

toe	16	24	604,784	seconds

IODE	8		see note 3

*	Parameters so indicated are two's complement, with the sign bit (+ or -) occupying the MSB.

**	Unless otherwise indicated in this column, effective range is the maximum range attainable 	with indicated bit allocation and scale factor.

note 1:	The URA reported in the navigation message will correspond to the maximum value anticipated during each subframe fit interval with uniform SA levels invoked.  Referring to the decimal equivalent of the transmitted four-bit binary number as N; with N a positive integer in the range of 0 through 15; the accuracy value is defined to mean "no better than X meters", in accordance with the following relationships:

  If the value of N is 6 or less, X = 2(1+N/2),

  If the value of N is 6 or more, but less than 15, X = 2(N-2),

  N = 15 will indicate the absence of an accuracy prediction and will advise the SPS user to use that satellite at the user’s own risk,

For N = 1, 3 and 5; X is rounded to 2.8, 5.7 and 11.3 meters respectively; the above relationships yield integer values of X for all other values of N.  Using these values of X the user may utilize a look-up table approach for interpreting the URA message.

note 2:	The IODC indicates the issue number of the data set and thereby provides the user with a convenient means of detecting any change in the correction parameters.  The transmitted IODC will be different from any value transmitted by the satellite during the preceding seven days.

note 3:	The IODE is provided in both subframes 2 and 3 for the purpose of comparison with the 8 LSBs of the IODC term in subframe 1.  Whenever these three terms do not match, a data set cutover has occurred and new data must be collected.  The transmitted IODE will be different from any value transmitted by the satellite during the preceding six hours

### MESSAGE TYPE 30 - KLOBUCHAR DATA

The TRS station shall provide GPS broadcast Klobuchar single frequency Ionosphere model parameters to the TMS.  This data is formatted into a type 30 message.  This message contains the Klobuchar model parameters common to all GPS satellites.  Therefore the TRS sends this message to the TMS at the slow rate and polls the receiver for this information at a similar rate.

The message includes a header, the block of Klobuchar data and a CCITT CRC checksum.  The “Time of Validity” fields shall contain the same values as the “Time of Validity” of the last message type 1 sent.

The Klobuchar information in message type 30 shall use the same data scaling and resolution as defined in section 2.4.5.6 of the GPS-Standard Positioning Service Signal Specification document.

The following table details the structure of the message and the data size for each field:

| BYTE | DESCRIPTION | Size Bytes | Data type |
| --- | --- | --- | --- |
| 1 | Message type = 30 | 1 | U Char |
| 2-3 | Receiver ID | 2 | U Short |
| 4-5 | Time of Validity - GPS Week | 2 | U Short |
| 6-9 | Time of Validity - milliseconds | 4 | U Long |
|  | Klobuchar Data |  |  |
| 10-13 | Time Klobuchar data was received (Absolute GPS time in sec) | 4 | U Long |
| 14 | Alpha 0 Amplitude coefficient ( sec) | 1 | Char |
| 15 | Alpha 1 Amplitude coefficient ( sec/semi-circle) | 1 | Char |
| 16 | Alpha 2 Amplitude coefficient ( sec/semi-circle^2) | 1 | Char |
| 17 | Alpha 3 Amplitude coefficient ( sec/semi-circle^3) | 1 | Char |
| 18 | Beta 0 Period coefficient ( sec) | 1 | Char |
| 19 | Beta 1 Period coefficient ( sec/semi-circle) | 1 | Char |
| 20 | Beta 2 Period coefficient ( sec/semi-circle^2) | 1 | Char |
| 21 | Beta 3 Period coefficient ( sec/semi-circle^3) | 1 | Char |
| 22-23 | Checksum calculated on previous bytes - CCITT CRC | 2 | Short |

table 4 : message type 30

Overhead (Header + checksum) = 11 bytes

Length of message type 30 = 23 bytes

parameter	# of bits	scale	range **	units

0	8 *	2-30		seconds

1	8 *	2-27		sec/semi-circle

2	8 *	2-24		sec/semi-circle2

3	8 *	2-24		sec/semi-circle3

0	8 *	211		seconds

1	8 *	214		sec/semi-circle

2	8 *	216		sec/semi-circle2

3	8 *	216		sec/semi-circle3

*	Parameters so indicated are two's compliment, with the sign bit (+ or -) occupying the MSB

** Unless otherwise indicated in this column, effective range is the maximum range attainable with indicated bit allocation and scale factor.

### MESSAGE TYPE 31 - UTC DATA

The TRS station shall provide GPS broadcast UTC offsets and leap second data to the TMS.  This data is formatted into a message type 31.  This message contains the offset of GPS time from UTC (USNO) common to all GPS satellites.  Therefore the TRS transmits this message to the TMS at a slow rate and polls the receiver for this information at a similar rate.

The message includes a header, the block of UTC data and a CCITT CRC checksum.  The “Time of Validity” fields shall contain the same values as the “Time of Validity” of the last message type 1 sent.

The UTC information in message type 31 shall use the same data scaling and resolution as defined in section 2.4.5.5 of the GPS-Standard Positioning Service Signal Specification document.

The following table details the structure of the message and the data size for each field:

| BYTE | DESCRIPTION | Size Bytes | Data Type |
| --- | --- | --- | --- |
| 1 | Message type = 31 | 1 | U Char |
| 2-3 | Receiver ID | 2 | U Short |
| 4-5 | Time of Validity - GPS Week | 2 | U Short |
| 6-9 | Time of Validity - milliseconds | 4 | U Long |
|  | UTC Data: |  |  |
| 10-13 | A0 UTC offset coefficient (sec) | 4 | Long |
| 14-17 | A1 UTC offset coefficient (sec/sec) | 4 | Long |
| 18 | DtLS Delta time due to leap seconds (sec) | 1 | U Char |
| 19 | tot Reference time for UTC data (sec) | 1 | U Char |
| 20 | WNt UTC reference week number (weeks) | 1 | U Char |
| 21 | WNLSF Effectivity reference week number (weeks) | 1 | U Char |
| 22 | DN Effectivity reference day number (days) | 1 | U Char |
| 23 | DtLSF Delta time at leap second event (sec) | 1 | Char |
| 24-25 | Checksum CCITT CRC | 2 | Short |

table 5 : message type 31

Overhead (Header + Checksum) = 11 bytes

Length of message 31 = 25 bytes

parameter	# of bits	scale	range ***	units

A0	32 *	2-30		seconds

A1	24 *	2-50		seconds/sec

tLS	8	1		seconds

tot	8	212	602,122	seconds

WNt	8	1		weeks

WNLSF 	8	1		weeks

DN	8 **	1	7	days

tLSF	8	1		seconds

*	Parameters so indicated are two's compliment, with the sign bit (+ or -) occupying the MSB.

**	Right Justified.

***	Unless otherwise indicated in this column, effective range is the maximum range attainable with indicated bit allocation and scale factor.

### MESSAGE TYPE 32 - ALMANAC DATA

The TRS station shall provide GPS broadcast almanac data for all of the satellites in the GPS constellation.  The data for all satellites is formatted into a message type 32. This message contains the almanac data for all operational GPS satellites.  Therefore the TRS sends this message to the TMS at a slow rate and polls the receiver for this information at a similar rate.

NOTE: ICD was amended. There is now one message type 32 sent for each satellite. Byte 12 is now always set to 1.

The message includes a header, the block of Almanac data and a CCITT CRC checksum.  The “Time of Validity” fields shall contain the same values as the “Time of Validity” of the last message type 1 sent.

The Almanac information in message type 32 shall use the same data scaling and resolution as defined in section 2.4.5.2.2 of the GPS-Standard Positioning Service Signal Specification document.

| BYTE | DESCRIPTION | Size Bytes | Data Type |
| --- | --- | --- | --- |
| 1 | Message type = 32 | 1 | U Char |
| 2-3 | Receiver ID | 2 | U Short |
| 4-5 | Time of Validity - GPS Week | 2 | U Short |
| 6-9 | Time of Validity - milliseconds | 4 | U Long |
| 10 | DWN Difference between GPS Week and Week of almanac | 1 | Char |
| 11 | toa Time of almanac | 1 | U Char |
| 12 | Number of satellites in the record | 1 | U Char |
|  | Almanac Data: 28 Bytes * number of SVs |  |  |
|  | Block repeated for each satellite in the record |  |  |
| n+ |  |  |  |
| 1 | Satellite PRN number | 1 | U Char |
| 2-3 | e eccentricity | 2 | U Short |
| 4-5 | di Delta inclination from 0.3 (semi-circles) | 2 | Short |
| 6-7 | OMEGADOT Rate of right ascension (semi-circles/sec) | 2 | Short |
| 8-11 | (A)1/2 Square root of semi-major axis (meters1/2) | 4 | U Long |
| 12-15 | (OMEGA)0 Longitude of ascending node (semi-circles) | 4 | Long |
| 16-19 | w Argument of perigee (semi-circles) | 4 | Long |
| 20-23 | M0 Mean anomaly at reference time (semi-circles) | 4 | Long |
| 24-25 | af0 Clock data coefficient (seconds) | 2 | Short |
| 26-27 | af1 Clock data coefficient (seconds/sec) | 2 | Short |
| 28 | Health | 1 | U Char |
| 2 last | Checksum CCITT CRC | 2 | Short |

table 6 : message type 32

parameter	# of bits	scale	range **	units

WN	8 *	1		weeks

toa	8	212	602,112	seconds

Num satellites	8	1	32	dimensionless

e	16	2-21		dimensionless

i	16 *	2-19		semi-circles

OMEGADOT	16 *	2-38		semi-circles/sec

(A)1/2	24	2-11		meters1/2

(OMEGA)0	24 *	2-23		semi-circles

	24 *	2-23		semi-circles

M0	24 *	2-23		semi-circles

af0	11 *	2-20		seconds

af1	11 *	2-38		seconds/sec

Health				see Note 1

*	Parameters so indicated are two's compliment, with the sign bit (+ or -) occupying the MSB.

**	Unless otherwise indicated in this column, effective range is the maximum range attainable with indicated bit allocation and scale factor.

Note 1:  This eight bit word matches bits 17 through 24 of word five in the 32 pages which contain almanac data for the individual satellites.

#### MESSAGE TYPE 40: TMS-to-TUS MESSAGE

- This message carries the WAAS message that has been formatted by the TMS for broadcast by the GEO. The TMS shall send three (3) consecutive Message Type 40s each second (each containing an identical WAAS message) to each available TUS. The message sequence counter shall be set to zero, one, and two for each of these messages, respectively, to differentiate between them.   The TMS Availability flag contained in the Message Type 40 is configured by operator command and shall default to not available.  Each Message Type 40 message includes a header, the 250 bit WAAS message, and a CCITT CRC checksum.

- NOTE: Message Type 40 is only sent once for VHF operations

- The Time of Validity represents the time the first bit of the WAAS message should be transmitted from the GEO satellite (top of a GPS second). Thirty-two (32) bytes are used to store the 250 bit WAAS message. The last six (6) unused bits shall be null.  The Message Type 40 shall arrive at the TUS at least 400 msec prior to the time of validity of the WAAS message.

- The following table details the structure of message and the data sizes reserved for each field:

| - BYTE | - DESCRIPTION | - Size - bytes | - Data type |
| --- | --- | --- | --- |
| - 1 | - Message type = 40 | - 1 | - U Char |
| - 2-3 | - TMS ID | - 2 | - U Short |
| - 4-5 | - Time of Validity - GPS week | - 2 | - U Short |
| - 6-9 | - Time of Validity - milliseconds (time of week) | - 4 | - U Long |
| - 10 | - Message sequence counter (0, 1, or 2) | - 1 | - U Char |
| - 11 | - GEO PRN | - 1 | - U Char |
| - 12-13 | - TMS status flags (see bit field table) | - 2 | - U Short |
| - 14-45 | - WAAS message (250 bits) | - 32 | - Char |
| - 46-47 | - Checksum calculated on previous bytes - CCITT CRC | - 2 | - Short |

- table 7 : Message Type 40

- The status bit field for Message Type 40 is described in the following table :

| - Bit | - Description | - Value |
| --- | --- | --- |
| - 0 | - Availability Designation | - 1 = available for broadcast - 0 = unavailable for broadcast |
| - 1-15 | - Reserved | - unused bits are null |

- Table 8 : Message Type 40 TMS Status Bit Fields

- Overhead (header + checksum): 11 bytes

Total length of the message: 47 bytes

#### MESSAGE TYPE 41 : TUS-to-TMS MESSAGE

NOTE: Message Type 41 is not implemented for VHF operations

- Message Type 41 returns from the TUS to the TMS, a copy of the WAAS message broadcasted by the GEO, along with satellite tracking and status data. The satellite tracking data is from the GSV receiver that tracks both GPS satellites and the GEO. The TUS shall send a Message Type 41 to each TMS every second. Each message includes a header, message timing information, GEO tracking data, the FEC decoded WAAS message broadcast by the GEO, GPS tracking data, and a CCITT CRC checksum.

- The Time-of-Validity represents the time that the first bit of the WAAS message is broadcast from the GEO. The TMS ID and message sequence counter from the Message Type 40 used for the WAAS message broadcast to the GEO are included in this message.

- Thirty-two (32) bytes are used to store the FEC decoded WAAS message (250 bits). The last 6 unused bits shall be null.

| - BYTE | - DESCRIPTION | - Size - bytes | - Data type |
| --- | --- | --- | --- |
| - 1 | - Message type = 41 | - 1 | - U Char |
| - 2-3 | - TUS ID | - 2 | - U Short |
| - 4-5 | - Time of Validity - GPS week | - 2 | - U Short |
| - 6-9 | - Time of Validity - milliseconds (time of week) | - 4 | - U Long |
| - 10-11 | - TUS status flags (see bit field table below) | - 2 | - U Short |
| - 12 | - Number of TMSs sending Message Type 40s | - 1 | - U Char |
| - 13 | - Number of GPS satellites tracked | - 1 | - U Char |
| - 14-17 | - Time first bit of WAAS message broadcast to GEO - milliseconds (time of week) | - 4 | - U Long |
| - 18-21 | - Time last bit of WAAS message received from GEO - milliseconds (time of week) | - 4 | - U Long |
|  | - GEO satellite data: 77 bytes |  |  |
| - 22 | - GEO PRN number | - 1 | - U Char |
| - 23-54 | - FEC decoded WAAS message received from GEO (250 bits) | - 32 | - Char |
| - 55-62 | - L1 pseudorange (meters) | - 8 | - Double |
| - 63-70 | - L1 carrier range (meters) | - 8 | - Double |
| - 71-74 | - L1 Doppler (m/s) (Hz?) | - 4 | - Float |
| - 75-78 | - L1 frequency correction (meters/sec) (Hz?) | - 4 | - Float |
| - 79-82 | - L1 signal-to-noise ratio (SNR) (dB-Hz) | - 4 | - Float |
| - 83-90 | - Computed geometric range (meters) | - 8 | - Double |
| - 91-94 | - Computed ionospheric delay (meters) | - 4 | - Float |
| - 95-98 | - Computed tropospheric delay (meters) | - 4 | - Float |
|  | - TMS data: 9 bytes |  |  |
| - n+ | - Repeated for each TMS |  |  |
| - 1-2 | - TMS ID | - 2 | - U Short |
| - 3-4 | - TMS status flags (see bit field table) | - 2 | - U Short |
| - 5 | - Message sequence counter of Message Type 40 (0, 1, or 2) | - 1 | - U Char |
| - 6-9 | - Time Message Type 40 was received from TMS - milliseconds (time of week) | - 4 | - U Long |
|  | - GPS satellite data: 29 bytes/channel |  |  |
| - n+ | - Repeated for each GPS satellite tracked |  |  |
| - 1 | - Satellite PRN number | - 1 | - U Char |
| - 2-5 | - Satellite status flags (see bit field table ) | - 4 | - U Long |
| - 6-13 | - L1 pseudorange (meters) | - 8 | - Double |
| - 14-21 | - L1 carrier range (meters) | - 8 | - Double |
| - 22-25 | - L1 Doppler (m/s) (Hz?) | - 4 | - Float |
| - 26-29 | - L1 signal-to-noise ratio (dB-Hz) | - 4 | - Float |
| - last 2 | - Checksum calculated on previous bytes - CCITT CRC | - 2 | - Short |

- Table 9 : Message Type 41

- The TUS status flags field for Message Type 41 is described in the following table :

| - Bit | - Description | - Value |
| --- | --- | --- |
| - 0 | - GEO Receiver subsystem status | - 1 = OK; 0 = Bad |
| - 1 | - 7201 subsystem status | - 1 = OK; 0 = Bad |
| - 2 | - GPS subsystem status | - 1 = OK; 0 = Bad |
| - 3-15 | - Reserved | - unused bits are null |

- table 10 : Message Type 41 TUS Status Bit Field

- The TMS status flags field for Message Type 41 is described in the following table :

| - Bit | - Description | - Value |
| --- | --- | --- |
| - 0 | - Availability Designation | - 1 = available for broadcast - 0 = unavailable for broadcast |
| - 1 | - Primary Status | - 1 = selected as primary - 0 = selected as backup |
| - 2-15 | - Reserved | - unused bits are null |

- Table 11: Message Type 41 TMS Status Bit Field

- The GPS satellite status flags for Message Type 41 is described in the following table:

| - Bit | - Description | - Value |
| --- | --- | --- |
| - 0 | - L1 Tracking mode | - 1 = P-code 0 = C/A code |
| - 1-3 | - L1 Cycle slip counter | - 0 to 7 |
| - 4-15 | - Reserved | - unused bits are null |
| - 16-31 | - Reserved for GPS receiver specific tracking status | - unused bits are null |

- table  12 : Message Type 41 GPS Satellite Status Bit Field

- Bits 16-31 are reserved for bits 0-15 of the GPSCard Tracking Status flag present in the Channel Range Measurement message of the NovAtel receiver.

- The total length of the message depends on the number of GPS satellites tracked.

- Overhead (header + checksum): 11 bytes

- Maximum length (11 GPS satellites, 4 TMSs): 455 bytes

Average length (8 GPS satellites, 2 TMSs): 350 bytes

#### MESSAGE TYPE 50: SQM DATA MESSAGE

- This message carries the Signal Quality Monitoring Data as provided by Novatel OEM4 receivers (with the right hardware and software configuration). One message is sent every second grouping all PRNs available and contains the SVM data for these satellites. Each Message Type 50 message includes a header, the SVM data and a CCITT CRC checksum.

- The message is sent when data for the next second has been received from the receiver for at least one PRN.

- The following table details the structure of message and the data sizes reserved for each field:

| - BYTE | - DESCRIPTION | - Size - bytes | - Data type |
| --- | --- | --- | --- |
| - 1 | - Message type = 50 | - 1 | - U Char |
| - 2-3 | - RID (Receiver ID) | - 2 | - U Short |
| - 4-5 | - Time of Validity - GPS week | - 2 | - U Short |
| - 6-9 | - Time of Validity - milliseconds (time of week) | - 4 | - U Long |
| - 10 | - Number of PRNs Following | - 1 | - U Char |
|  | - Next Block repeated For Each PRN |  |  |
| - 11 | - PRN | - 1 | - U Char |
| - 12 | - Number of Channels N for that PRN | - 1 | - U Char |
|  | - Next 6 lines repeated N times, 1 for each channel |  |  |
| - 13-16 | - Accumulation 1 (A1sum) | - 4 | - U Long |
| - 17-20 | - Accumulation 2 (A1sum) | - 4 | - U Long |
| - 21-24 | - Accumulation 3 (A1sum) | - 4 | - U Long |
| - 25-28 | - Accumulation 4 (A1sum) | - 4 | - U Long |
| - 29-32 | - Accumulation 5 (A1sum) | - 4 | - U Long |
| - 33 | - Sync Flag (0 = In Sync 1= Not sync) | - 1 | - U Char |
| - …… | - ………………… |  |  |
|  | - Checksum calculated on previous bytes - CCITT CRC | - 2 | - Short |

- table 11 : Message Type 50

- Overhead (header + checksum): 12 bytes

Total length of the message:

Max Length (?): 14 PRN, 2 Channels : 12 + 14*(2+2*21) = 628

#### MESSAGE TYPE 51: SQM INFO MESSAGE

- This message carries the Signal Quality Monitoring INFO as provided by Novatel OEM4 receivers (with the right hardware and software configuration). The message rate is set in the TRS software configuration file. All messages provided by the receiver are combined into one type 51 message. The message has a number of channels and for each channel, up to 2 set of channel data. The Time of validity is the taken from the last Type 1 sent. Each Message Type 51 message includes a header, the SVM info and a CCITT CRC checksum.

- The following table details the structure of message and the data sizes reserved for each field:

| - BYTE | - DESCRIPTION | - Size - bytes | - Data type |
| --- | --- | --- | --- |
| - 1 | - Message type = 51 | - 1 | - U Char |
| - 2-3 | - RID (Receiver ID) | - 2 | - U Short |
| - 4-5 | - Time of Validity - GPS week | - 2 | - U Short |
| - 6-9 | - Time of Validity - milliseconds (time of week) | - 4 | - U Long |
| - 10 | - Receiver Configuration 0=Narrow 1= UltraNarrow | - 1 | - U Char |
| - 11 | - Number of Channels N | - 1 | - U Char |
|  | - Repeated for each channel (up to N) |  |  |
| - 1 | - Channel Number | - 1 | - U Char |
| - 2-5 | - Accumulation Time in milliseconds | - 4 | - U Long |
| - 6 | - Number of Internal Channels for that Channel (Novatel calls everything a channel!) n (n<=2) | - 1 | - U Char |
|  | - Next block repeated n times, 1 for each internal channel |  |  |
| - 1-4 | - A1xval Correlator spacing, in C/A chips from punctual, for accumulation 1 * | - 1 | - Char |
| - 5-8 | - A2xval Correlator spacing for accumulation 2 * | - 1 | - Char |
| - 9-12 | - A3xval Correlator spacing for accumulation 3 * | - 1 | - Char |
| - 13-16 | - A4xval Correlator spacing for accumulation 4 * | - 1 | - Char |
| - 17-20 | - A5xval Correlator spacing for accumulation 5 * | - 1 | - Char |
| - 21-22 | - Status A | - 1 | - U Char |
| - 23-24 | - Status B | - 1 | - U Char |
| - …… | - ………………… |  |  |
|  | - Checksum calculated on previous bytes – CCITT CRC | - 2 | - Short |

- table 12 : Message Type 51

- Overhead (header + checksum): 13 bytes

Total length of the message (21 channels / 2 internal channels each): 13 + 21 * (6 + (2 * 7)) = 433

Status A:

Correlation Accumulation Types for A1 through A5

0x00 = All correlators correlating on Inphase samples

0x01 = A1 correlating on Quadrature samples

0x02 = A2 correlating on Quadrature samples

0x04 = A3 correlating on Quadrature samples

0x08 = A4 correlating on Quadrature samples

0x10 = A5 correlating on Quadrature samples

Status B:

Correlation Accumulation Types for A1 through A5

0x00 = All correlators correlating on full Early or Late samples

0x01 = A1 correlating on E-L samples

0x02 = A2 correlating on E-L samples

0x04 = A3 correlating on E-L samples

0x08 = A4 correlating on E-L samples

0x10 = A5 correlating on E-L samples

* Correlator spacing have been scaled by 0.025575

Ex: Char = -3     -> (Float) Correlator spacing = -3 * 0.025575 = -0.076725

### MESSAGE TYPE 100 – WAAS Subsystem Status DATA

The WEI provides subsystem status data.  This data is collected by the WEI server software and formatted into a Type 100 message.  The message type 100 shall be sent when WEI is providing data on the network.

The message includes a header, a block of bytes for each WAAS subsystem (status information) and a CCITT CRC checksum.  The fields of the header provide the number of WAAS subsystem status messages provided by the WEI.

The following table details the structure of message and the data sizes reserved the for each field:

| BYTE | DESCRIPTION | Size bytes | Data type |
| --- | --- | --- | --- |
| 1 | Message type = 100 | 1 | U Char |
| 2-3 | WEI ID | 2 | U Short |
| 4-5 | Time of Validity - GPS Week | 2 | U Short |
| 6-9 | Time of Validity – milliseconds | 4 | U Long |
| 10 | Number of WAAS subsystem status messages | 1 | U Char |
|  | Status data : 9 bytes/subsystem |  |  |
|  | Repeated for each WAAS subsystem |  |  |
| 11-12 | WAAS subsystem ID number | 2 | U Char |
| 13 | Network Status | 1 | U Char |
| 14 | Master Status Indication (mode) | 1 | U Char |
| 15 | Composite Fault Code Status | 1 | U Char |
| 16 | 43P Status | 1 | U Char |
| 17 | Receiver Status | 1 | U Char |
| 18 | Cesium Status | 1 | U Char |
| 19 | Power Control Unit (PCU) Status | 1 | U Char |
| 2 last | Checksum calculated on previous bytes - CCITT CRC | 2 | Short |

table 13 : message type 100

The Network status field is described in the following table:

| Network Status Value | Network Indication |
| --- | --- |
| 0 | Network 1 and Network 2 status message received this epoch |
| 1 | Network 1 status message received this epoch |
| 2 | Network 2 status message received this epoch |

The Master Status Indication field is described in the following table:

| Master Status Value | Master Status Indication (mode) |
| --- | --- |
| 1 | Inactive |
| 2 | Startup |
| 3 | Maintenance |
| 4 | Verification |
| 5 | Normal |
| 6 | Faulted |
| 7 | Backup (GUS only) |
| 8 | Primary (GUS only) |
| 9 | No data reported |

The Composite fault Code, 43P, Receiver, Cesium, and PCU status fields are described in the following table:

| Status Value | Status Indication |
| --- | --- |
| 0 | No error |
| 1 | Non-critical error |
| 2 | Critical error |